Automic on-off router

The relay I used is the same IoT Relay that several others have mentioned in FB groups. For those who are comfortable with controls terminology, the IoT Relay is a DPST interposing relay that has been integrated into a very convenient “black box” package with the following features:

 a main power switch,

 two normally open (NO) outlets (which the IoT calls “normally OFF”),

 one normally closed (NC) outlet (which the IoT calls “normally ON”),

 one “always ON” outlet (i.e., “always” means whenever the main power switch is on), and

 a control input that accepts a very wide range of input signals, 3-48VDC or 12-120VAC,

… all for less than $30. In short, you could probably save a couple of bucks by buying all of the components separately and assembling the package yourself, but I can’t imagine why you would want to (unless perhaps electronics is a hobby of yours!)

Here’s how it works:

a. connect the relay to the controller.

b. At the beginning of the program, our gcode needs to include the “M3” command, which tells the controller to energize the SpinPWM.

c. When the IoT senses that signal, it closes (i.e., turns on) the NO outlets and opens (i.e., turn off) the NC outlet.

d. The router (with its switch already turned on) is plugged into one of the NO outlets, so it starts running.

e. At the end of the program, our gcode needs to include the “M5” command, which tells the controller to de-energize the SpinPWM.

f. In the absence of the control signal the IoT Relay returns to its inital state, and our router stops turning.

Simple huh? Okay, I realize that I’m not the best at explaining this sort of stuff to people, so let me walk through the

process that I used, step by step.

Materials:

• IoT Relay https://www.amazon.com/Iot-Relay-Enclosed-High-Power-Raspberry/dp/B00WV7GMA2/ref=sr_1_2?crid=2X4QQUO651Y62&dchild=1&keywords=iot+power+relay&qid=1594415370&sprefix=iot+p%2Caps%2C390&sr=8-2

• Wire. This will be the wire that will be strung from the Controller to the IOT Relay.

I recommend something flexible that has stranded copper wire.

Once you’ve gathered up all the stuff you need, here are the installation steps:

1. Connect one end of the wires to the controller output for SpinPWM(green plug comes out of controller).  Use the supplied connector on the control board.  Take care to remember what color goes to ground and what color goes to SpinPWM.

2. Connect the other end of the wires to the IOT Relay (green plug comes out to expose screws). (-) to ground and (+) to SpinPWM

3. Ensure the wires are connected to correct ports and ensure green plugs are secure

4. Plug the router into one of the outlets marked “normally OFF”.

5. Plug the IoT Relay into a 120VAC outlet.

6. Turn on the IoT Relay’s power switch.

7. Turn on your router’s switch (I used the second Normally off switch for the vacuum).

16. Ensure that your soSware is providing the “M3” command near the beginning of the gcode file, and “M5” near the end.

In VCarve Desktop, I needed to edit the post processor file to insert the M3 and M5 commands.

I’m no expert on this stuff, but here is how I went about it and it is working

for me.

1- Open Vcarve

2- Go to file and click on "Open Application Data Folder"

3- Open folder labeled "PostP"

4- Find file name “Grbl_inch”, and Copy the file

5- Hit the back button

6- Open My_PostP and paste the "Grbl_inch" file

As the readme file in My_PostP explains, any post processors that are placed in that folder will be the only options displayed in the software. Since I only plan to use grbl inch, this sounded like a great plan to avoid accidentally grabbing the wrong file.

I opened the newly-copied Grbl_inch file in Notepad and added the M3 command to the header and the M5 command to the footer as shown below .

Scroll to bottom

Save

Any toolpath you have already created will need to be recreated in order to make this function work.  again not a pro so sorry if this is confusing